VPN CONNECTIVITY FORMS

| Required Data | Rancard VPN Information | Click Mobile VPN Information |
| --- | --- | --- |
| Device type and Version (e.g. PIX 6.1, CheckPoint FW-1, Netscreen ,etc) | Cisco ASA 5508 Sec+ | FortiGate 61E v6.0.2 |
| External peering IP address | 67.192.152.24 | 216.55.137.19 |
| External peering port/s | ICMP | ICMP |
| Encryption Domain (IP address range to be encrypted) | 172.24.32.54 172.24.96.42, 172.24.32.52 | 10.199.0.10 |
| Ping able address (within the VPN) |  | 10.199.0.10 |
| Secure connection setup protocol | IPSEC | IPSEC |
| Phase 1 - Encryption algorithm (e.g. DES, 3DES, etc) | AES256 | AES256 |
| Phase 1 - Hashing algorithm (e.g. MD5, SHA1) | SHA2 | SHA2 |
| Phase 2 - Encryption algorithm (e.g. DES, 3DES, etc) | AES256 | AES256 |
| Phase 2 - Hashing algorithm (e.g. MD5, SHA1) | SHA2 | SHA2 |
| Authentication String (pre-shared key or CA) | To be shared via sms |  |
| Diffe-Hellman Group (1,2) | Group 14 | Group 14 |
| IKE SA Lifetime (Phase1) | 480 | 480 |
| IPsec SA Lifetime (Phase2) | 480 | 480 |
| Technical contact |  |  |

##### Requestor/Customer Authorization

Authorized Signature: 	  Date:

Printed Name: