# Interface Specification

WAP

Interface Diameter

Filename: NGIN_MASCOM_COEI_INTERFACE_DIAMETER_WAP

Version History

| Version | Date | Responsibilities | Subject/Comment |
| --- | --- | --- | --- |
| 1.0.0 | 2014-03-04 | Edition: Rita Catarina Gaspar; Renato Peixe | First version |
|  | 2014-03-04 | Revision: João Ribeiro da Silva, Renato Peixe, Paulo Balão |  |
|  | 2014-03-04 | Approval: Rita Catarina Gaspar |  |
| 1.0.1 | 2014-03-17 | Edition: Rita Catarina Gaspar | Update document with the AVP service parameters category and content id for MIVR events. |
|  | 2014-03-17 | Revision: Renato Peixe; João Ribeiro da Silva |  |
|  | 2014-03-18 | Approval: Rita Catarina Gaspar |  |

1 .	Introduction	3

1.1 .	Architecture	3

1.2 .	Terms and Definitions	3

1.3 .	Legend	4

2 .	Event Based Charging	4

2.1 .	CCR ( Credit - Control - Request)	4

2.2 .	CCA ( Credit - Control - Answer)	7

2.2.1	Result Codes	9

3 .	Event Flow	9

3.1 .	Reserve/Debit	9

# Introduction

This document contains the interface specification of the Diameter Credit Control Application (DCCA) for integration of the WAP-GW Platform and NGIN Platform.

The chosen protocol for the real-time interface between the WAP-GW and NGIN Platform  is the Diameter Credit Control Application (DCCA). It is a standard Diameter-base application, as used in the telecommunication market for the implementation of real-time credit control and as a service charge interface between a network element and a billing system.

The definition of this interface is based on 3GPP 32.299 [4] and RFC 4006 [2].

Note that this integration involves the service charge of all the MySpot and Mobile Radio IVR events.

## Architecture

In the figure below we can see an architecture diagram proposed for this diameter integration:

WAP-GW Diameter Platform

DSGW

DSCF

DSCP

SLR

SDF

SL-DIAM

SDP

Fig. 1 – Logical Architecture

## Terms and Definitions

Following is the list of terms that can be used in this document that the reader should be familiar with:

| Term | Definition |
| --- | --- |
| AO | Application Originated |
| AT | Application Terminated |
| AVP | Attribute Value Pairs |
| DCCA | Diameter Credit-Control Application |
| IEC | Immediate Event Charging |
| MO | Mobile Originating |
| MT | Mobile Terminating |
| OCS | Online Charging |
| SPS | Subscriber Provisioning System |
| CCR | Credit-Control-Request |
| CCA | Credit-Control-Answer |

## Legend

AVP field Categories (Request):

- Mandatory (M): AVP that WAP-GW platform will include

- Optional (O): AVP that WAP-GW platform will NOT include

- Conditional (C): AVP WAP-GW platform will include, if available.

AVP field Categories (Response):

- Mandatory (M): AVP that WAP-GW platform requires

- Optional (O): AVP that  WAP-GW platform ignores if received.

# Event Based Charging

The charging model will be based on Reserve/Release or Reserve/Debit pair of interactions with the NGIN Platform.

The WAP-GW platform will invoke the NGIN via Diameter interface to charge a service.

The Diameter interface will send Credit-Control-Request (CCR) messages to NGIN, that will then perform the appropriate operation (Reserve, Release Reserve or Debit) according to {Requested-Action} attribute value in the CCR message. After processing the request, NGIN platform will answer with a Credit-Control-Answer message to the WAP-GW platform.

The specification of the messages and the meaning and values of the messages attributes are specified below.

| Command-Name | Abbr. | Code | Direction |
| --- | --- | --- | --- |
| Credit-Control-Request | CCR | 272 | Client (Diameter Platform) → Server (IN Platform) |
| Credit-Control-Answer | CCA | 272 | Server (IN Platform) → Client (Diameter Platform) |

## CCR ( Credit - Control - Request)

The CCR message is used between the Diameter Credit Control and the NGIN Platform to request debit/charging authorization for a given service. There is a reliable authentication mechanism between the server and the clients.

The CCR Message format is based on that defined in the Techinical Specification of 3GPP TS 32.299.

Message Format

<CCR> ::= < Diameter Header: 272, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

[ Destination-Host ]

{ Destination-Realm }

{ Auth-Application-Id }

{ Service-Context-Id }

{ CC-Request-Type }

{ CC-Request-Number }

[ Event-Timestamp }

{ Subscription-Id }

{ Service-Identifier }

{ Requested-Action }

{ Requested-Service-Unit }

[ Service-Information ]

Note

The meaning of the AVP symbols are as follows:

<AVP> indicates that the AVP is mandatory and the location of the AVP in the message is fixed.

{AVP} indicates that the AVP is mandatory.

[AVP] indicates that the AVP is optional.

*AVP indicates that the AVP might be the same as some other AVPs.

| Nº | Diameter AVP | AVP Code | AVP Type | Option | Value | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Session-Id | 263 | UTF8String | M | Diameter session ID |  |
| 2 | Originator Host | 264 | DiamIdent | M | Host that sends Diameter messages, that is, the WAP-GW host. |  |
| 3 | Origin-Realm | 296 | DiamIdent | M | Domain of the host that sends Diameter messages, that is, the domain of the WAP-GW host. |  |
| 4 | Destination-Host | 293 | DiamIdent | M | Destination host, that is, the OCS. |  |
| 5 | Destination-Realm | 283 | DiamIdent | M | Domain of the destination host, that is, the domain of the OCS CBS. |  |
| 6 | Auth-Application-Id | 258 | Unsigned32 | M | Unique flag for re-authentication. | The value is 4 and is fixed |
| 7 | Service-Context-Id | 461 | UTF8String | M | Unique flag (expansible) of a DCC service | The value is WAPGW for charging services from MySpot. The value is MIVR for charging services from Mobile Radio IVR. |
| 8 | CC-Request-Type | 416 | Enumerated | M | Request type | The values are as follows: 1: INITIAL_REQUEST 3:TERMINATION_REQUEST |
| 9 | CC-Request-Number | 415 | Unsigned32 | M | Serial number of a request for uniquely identifying a request of the Diameter session |  |
| 10 | Event-Timestamp | 55 | Time | M | Event occurrence time | Defined on RFC 4006 |
| 11 | Subscription-Id | 443 | Grouped | M | Flag for identifying the information about a service subscriber. |  |
| 11.1 | Subscription-Id-Type | 450 | Enumerated | M | END_USER_E164: 0 END_USER_IMSI: 1 END_USER_SIP_URI: 2 END_USER_NAI: 3 END_USER_PRIVATE: 4 | END_USER_E164 (0) |
| 11.2 | Subscription-Id-Data | 444 | UTF8String | M | Subscriber flag | MSISDN (without a country code) |
| 12 | Requested-Service-Unit | 437 | Grouped | M | Total requested amount |  |
| 12.1 | CC-Money | 413 | Grouped | M | Total amount in the specified currency |  |
| 12.1.1 | Unit-Value | 445 | Grouped | M | Combination of the values of Value-Digits and Exponent to form a number containing an index. |  |
| 12.1.1.1 | Value-Digits | 447 | Integer64 | M |  | For example, when the amount is 0.05, this value is 5. |
| 12.1.1.2 | Exponent | 429 | Integer32 | O |  | For example, when the amount is 0.05, this value is -2. |
| 12.1.2 | Currency-Code | 425 | Unsigned32 | M | Currency code (specified in ISO4217) |  |
| 13 | Service-Parameter-Info | 440 | Grouped | O |  |  |
| 13.1 | Service-Parameter-Type | 441 | Unsigned32 | O | 19002 | This field is used in the WAPGW and MIVR events. The value is 19002 and is fixed. |
| 13.2 | Service-Parameter-Value | 442 | OctetString | O | Content id | This field is used in the WAPGW and MIVR events. |
| 13.2 | Service-Parameter-Type | 441 | Unsigned32 | O | 19003 | This field is used in the WAPGW. The value is 19003 and is fixed. |
| 13.4 | Service-Parameter-Value | 442 | OctetString | O | Content Name | This field is used in the WAPGW. |
| 13.5 | Service-Parameter-Type | 441 | Unsigned32 | O | 19006 | This field is used in the WAPGW The value is 19006 and is fixed. |
| 13.6 | Service-Parameter-Value | 442 | OctetString | O | Content Type code | This field is used in the WAPGW. |
| 13.7 | Service-Parameter-Type | 441 | Unsigned32 | O | 19007 | This field is used in the WAPGW The value is 19007 and is fixed. |
| 13.8 | Service-Parameter-Value | 442 | OctetString | O | Supplier_code | This field is used in the WAPGW. |
| 13.9 | Service-Parameter-Type | 441 | Unsigned32 | O | 19008 | This field is used in the WAPGW The value is 19008 and is fixed. |
| 13.10 | Service-Parameter-Value | 442 | OctetString | O | Deliver to | This field is used in the WAPGW. |
| 13.11 | Service-Parameter-Type | 441 | Unsigned32 | O | 19009 | This field is used in the WAPGW The value is 19009 and is fixed. |
| 13.12 | Service-Parameter-Value | 442 | OctetString | O | iscoverymethod | This field is used in the WAPGW. |
| 13.13 | Service-Parameter-Type | 441 | Unsigned32 | O | 19105 | This field is used in the WAPGW and MIVR. The value is 19105 and is fixed. |
| 13.14 | Service-Parameter-Value | 442 | OctetString | O | category | This field is used in the WAPGW and MIVR. |

## CCA ( Credit - Control - Answer)

The CCA (Credit-Control-Answer) message is used to send charging requests from the server to clients. There is a reliable authentication mechanism between the server and the clients.

Message Format

<CCA>::= < Diameter Header: 272, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

{ Auth-Application-Id }

{ CC-Request-Type }

{ CC-Request-Number }

[ User-Name ]

[ CC-Session-Failover ]

[ Origin-State-Id ]

[ Event-Timestamp ]

{ Granted-Service-Unit }

Note

The meanings of the AVP symbols are as follows:

<AVP> indicates that the AVP is mandatory and the location of the AVP in the message is fixed.

{AVP} indicates that the AVP is mandatory.

[AVP] indicates that the AVP is optional.

*AVP indicates that the AVP might be the same as some other AVPs.

| Nº | Diameter AVP | AVP Code | AVP Type | Option | Value | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Session-Id | 263 | UTF8String | M | Diameter session ID |  |
| 2 | Result-Code | 268 | Unsigned32 | M | Result of the specific query. | Defined on RFC 3588 [401] |
| 3 | Origin-Host | 264 | DiamIdent | M | Host that sends Diameter messages, that is, the WAP-GW host. |  |
| 4 | Origin-Realm | 296 | DiamIdent | M | Domain of the host that sends Diameter messages, that is, the domain of the WAP-GW host. |  |
| 5 | Auth-Application-Id | 258 | Unsigned32 | M | Unique flag for re-authentication. | The value is 4 and is fixed |
| 6 | CC-Request-Type | 416 | Enumerated | M | Request type | The values are as follows: 1: INITIAL_REQUEST 3:TERMINATION_REQUEST |
| 7 | CC-Request-Number | 415 | Unsigned32 | M | Serial number of a request for uniquely identifying a request of the Diameter session |  |
| 8 | User-Name | 1 | UTF8String | O | User name in the format of a NAI | Defined on RFC 3588 [401] |
| 9 | CC-Session-Failover | 418 | Enumerated | O | Operation Failover |  |
| 10 | Origin-State-Id | 278 | Unsigned32 | O | State associated to the CTF. |  |
| 11 | Event-Timestamp | 55 | Time | O | Event occurrence time | Defined on RFC 4006 |
| 12 | Granted-Service-Unit | 431 | Grouped | M | The amount of granted service units for a particularcategory. |  |
| 12.1 | CC-Money | 413 | Grouped | M |  |  |
| 12.1.1 | Unit-Value | 445 | Grouped | M |  |  |
| 12.1.1.1 | Value-Digits | 447 | Integer64 | M |  | For example, when the amount is 0.05, this value is 5. |
| 12.1.1.2 | Exponent | 429 | Integer32 | O |  | For example, when the amount is 0.05, this value is -2. |
| 12.1.2 | Currency-Code | 425 | Unsigned32 | O | Currency code (specified in ISO4217) |  |
| 13 | Validity-Time | 448 | Unsigned32 | M |  |  |

### Result Codes

The result codes returned from the server to client in the CCA message will be the following ones:

| Value | Description |
| --- | --- |
| 2001 | OK -The credit adjustment is successful. The transaction has been processed successfully. |
| 4012 | NOK – Destination subscriber has not sufficient funds. |
| 5002 | NOK – Invalid Session Id |
| 5003 | NOK – Subscriber is in an incompatible state |
| 5004 | NOK – Invalid Parameter |
| 5012 | NOK – Undefined Error |
| 5030 | NOK – Subscriber does not exist. |
| 5031 | NOK – Rating Error |

# Event Flow

## Reserve/Debit

In the following scenario, the WAP GW Platform (CTF) request the NGIN (OCS) to assure the reservation of an amount of the specified number of monetary units from the subscriber's plafonds. A plafond debit operation that triggers the deduction of the amount from the subscriber's plafond is carried out following the conclusion of service delivery.

Fig. 2 – Event charging with reservation